WASTEWATER TREATMENT

Primary treatment involves:

1. Screening- to remove large objects, such as stones or sticks that could plug lines or block tank inlets.

2. Grit chamber- slows down the flow to allow grit to fall out

3. Sedimentation tank (settling tank or clarifier) settleable solids settle out and are pumped away, while oils float to the top and are skimmed off

Secondary treatment typically utilizes biological treatment processes, in which microorganisms convert non-settleable solids to settleable solids. Sedimentation typically follows, allowing the settleable solids to settle out. Three options include:

1. Activated Sludge- The most common option uses microorganisms in the treatment process to break down organic material with aeration and agitation, then allows solids to settle out. Bacteria-containing "activated sludge" is continually recirculated back to the aeration basin to increase the rate of organic decomposition.

2. Trickling Filters- These are beds of coarse media (often stones or plastic) 3-10 ft. deep. Wastewater is sprayed into the air (aeration), then allowed to trickle through the media. Microorganisms attached to and growing on the media, break down organic material in the wastewater. Trickling filters drain at the bottom; the wastewater is collected and then undergoes sedimentation.

3. Lagoons- These are slow, cheap, and relatively inefficient, but can be used for various types of wastewater. They rely on the interaction of sunlight, algae, microorganisms, and oxygen (sometimes aerated).

After primary and secondary treatment, municipal wastewater is usually disinfected using chlorine (or other disinfecting compounds, or occasionally ozone or ultraviolet light). An increasing number of wastewater facilities also employ tertiary treatment, often using advanced treatment methods. Tertiary treatment may include processes to remove nutrients such as nitrogen and phosphorus, and carbon adsorption to remove chemicals. These processes can be physical, biological, or chemical.

Settled solids (sludge) from primary treatment and secondary treatment settling tanks are given further treatment and undergo several options for disposal.

TERMS

Activated sludge: sludge particles produced by the growth of microorganisms in aerate tanks as a part of the activated sludge process to treat wastewater.

Aeration: exposing to circulating air; adds oxygen to the wastewater and allows other gases trapped in the wastewater to escape (the first step in secondary treatment via activated sludge process).

Biochemical oxygen demand (BOD): a laboratory measurement of wastewater that is one of the main indicators of the quantity of pollutants present; a parameter used to measure the amount of oxygen that will be consumed by microorganisms during the biological reaction of oxygen with organic material.

Biosolids: sludge that is intended for beneficial use. Biosolids must meet certain government- specified criteria depending on its use (e.g., fertilizer or soil amendment).

Decomposition: the process of breaking down into constituent parts or elements.

Domestic wastewater: wastewater that comes primarily from individuals, and does not generally include industrial or agricultural wastewater.

Effluent: treated wastewater, flowing from a lagoon, tank, treatment process, or treatment plant.

Grit chamber: a chamber or tank used in primary treatment where wastewater slows down and heavy, large solids (grit) settle out and are removed.

Influent: wastewater flowing into a treatment plant.

Lagoons (oxidation ponds or stabilization ponds): a wastewater treatment method that uses ponds to treat wastewater. Algae grow within the lagoons and utilize sunlight to produce oxygen, which is in turn used by microorganisms in the lagoon to break down organic material in the wastewater. Wastewater solids settle in the lagoon, resulting in effluent that is relatively well treated, although it does contain algae.

Municipal: of or relating to a municipality (city, town, etc.). Municipal wastewater is primarily domestic wastewater.

Primary treatment: the first stage of wastewater treatment that removes settleable or floating solids only; generally removes 40% of the suspended solids and 30-40% of the BOD in the wastewater.

Secondary treatment: a type of wastewater treatment used to convert dissolved and suspended pollutants into a form that can be removed, producing a relatively highly treated effluent. Secondary treatment normally utilizes biological treatment processes (activated sludge, trickling filters, etc.) followed by settling tanks and will remove approximately 85% of the BOD and TSS in wastewater. Secondary treatment for municipal wastewater is the minimum level of treatment required by the Clean Water Act.

Sedimentation: the process used in both primary and secondary wastewater treatment, that takes place when gravity pulls particles to the bottom of a tank (also called settling).

Settling tank (sedimentation tank or clarifier): a vessel in which solids settle out of water by gravity during wastewater or drinking water treatment processes.

Sludge: any solid, semisolid, or liquid waste that settles to the bottom of sedimentation tanks (in wastewater treatment plants or drinking water treatment plants) or septic tanks.

Tertiary treatment: any level of treatment beyond secondary treatment, which could include filtration, nutrient removal (removal of nitrogen and phosphorus) and removal of toxic chemicals or metals; also called "advanced treatment" when nutrient removal is included.

Total suspended solids (TSS): a laboratory measurement of the quantity of suspended solids present in wastewater that is one of the main indicators of the quantity of pollutants present.

Trickling filter process: a biological treatment process that uses coarse media (usually rock or plastic) contained in a tank that serves as a surface on which microbiological growth occurs. Wastewater trickles over the media and microorganisms remove the pollutants (BOD and TSS). Trickling filters are followed by settling tanks to remove microorganisms that wash off or pass through the trickling filter media.

Turbidity: the cloudy or muddy appearance of a naturally clear liquid caused by the suspension of particulate matter.

Wastewater: water that has been used for domestic or industrial purposes.